JEC SUBSONIC WIND TUNNEL

TEST PLAN SUMMARY

Preliminary Formula One Wind Tunnel Test Plan

Target Test Window:    April 18th 2008 & April 22nd 2008

Rev 1

Note:  Tunnel configuration changes from baseline highlighted with bold italics.

Test Advocacy – Faculty Sponsor: Prof. M. Amitay. Funded coursework as part of Senior Capstone Air Vehicle Design course.  These tests are needed for comparison of a variety of wing configurations, in addition to obtaining aerodynamic characteristics for stability analysis.

Test Objectives –  To obtain lift and drag polars, as well as aerodynamic constants for stability analysis for the Formula One wind tunnel model using 3 different wing tip configurations by recording 6-component force data for each configuration.  The Formula One model with replaceable wingtips will be run at varying angles of attack.  Additionally, flow visualizations will be obtained for the 3 different wing tip configurations at different angles of attack.

Schedule –  Tunnel availability from 8 AM to 4 PM on Friday, April 18th  and alternately on Tuesday, April 22nd from 8 AM to 4PM.

Configurations – One Formula One model with various wing tip configuration attachments.  A 3 inch sting riser will be used to reduce model interaction with strut/sting.

Test Matrix – There are 3 envisioned phases of testing:

(1)  A mapping of Formula One characteristics with three different wing tip configurations.  6-component force data will be collected at a velocity of 100 fps while varying the angle of attack from -6 degrees up to stall.

(2)  A mapping of Formula One characteristics with three different wing tip configurations.  6-component force data will be collected at a velocity of 100 fps while varying the side-slip angle from 0 degrees up to 10 degrees.

(3) Flow visualization on model run at 15 fps varying the angle of attack from -6 degrees to stall.

See attached excel spreadsheet for Test Matrix.

For phases one and two the basic forces and moments will be all that will be calculated.

Measurement Requirements –Force measurements will be taken manually in a student generated spreadsheet using the JEC 2220 6-component balance and on-site computer at the start and end of each tunnel run for zeros.  Signal/noise (S/N) ratio in data will be estimated graphically in real time.  S/N ratio should be at least 3 for a successful test (for primary indicators -  shear stress and roll moment).  Tunnel operating parameters such as pressure, temperature and velocity to be measured by JEC 2220 lab base indicators.  (See below).

Tunnel Operating Plan:  Speed control via base tunnel closed loop speed control – using pressure rings to avoid interference from wake of pitot tube.  Angle of attack adjustments to be using computer interface in on-site computer verified with tunnel off.  Progressive speed increments to verify model integrity analysis.

Conceptual Model Design and force estimates – JEC 2220/MDL  fabricated Formula One with replaceable wingtips, See risk mitigation section for preliminary force estimates.

Data Acquisition and Reduction Requirements – Team members will manually record six components of force and moments acting on the model using JEC 2220 base instruments.  Data reduction will be done outside wind tunnel lab.

Acceptance Criteria –Lift and drag coefficient should be similar to NACA 65a210 airfoil data through AOA regime.

Roles and Responsibilities – At all times it will be required to have at least two students in the lab facility at all times.  Student 1 will handle wind tunnel control and sting positioning via the interface computer.  Student 2 will handle force acquisition through computer supplied in JEC 2220 lab.

Resources – JEC 2220 to provide a wind tunnel with a 2 foot by 2 foot test section, six component force balance, and base tunnel speed/operating indicators and controls.  JEC 2220 to also provide the sting base with angle of attack adjustment mechanism.

Risks and Mitigation Plans – Formula One  Aerodynamic Center is axially approximately on balance center; AC is ~ 4.50” (+ .25/-0.5”) above balance center.   Formula One anticipated operating speed is 100 fps.  Using a Clmax (Abbot and Doenhoff for NACA 65a210  neglecting taper and  wing tip effects except in Sw) of 1.4 and a reference area of 37.125 square inches, this gives a maximum lift of 4 lbs and estimated roll moment of 0 inch-lbs due to lack of ailerons.  Estimated pitching moments of <= +/- 10 in-lb anticipated.  All forces expected to be well within balance capability.

Progressive speed and angle of attack test plan to incrementally verify model integrity and force estimates.  Speeds will be started at 30 ft/s and will be increased in 15 ft/s increments up to 100 ft/s.  Angle of attack will test at nominal zero, and progress to 18 degrees.

Safety Considerations:  Standard Lab protocols apply.

Change to Control Procedures – Once approved, test matrix (V, AOA,  wingtip attachment configurations) may be altered within boundaries of approved matrix limits on each variable at the direction of Dr. Amitay as early results indicate and time permit.  Other changes subject to approval by Dr. Amitay.

Status and Reporting Requirements –  Student responsibility.    Raw data must be reported in final report for Formula One air racer.

Test Evaluation Procedures – Student responsibility.  Data from the experiments will be used to apply to the development of the Formula One Air racer.

Plan Sponsor___________________________

Facility Approval_______________________